Compute the gradient of a scalar function and return grids of the two horizontal

components that conform with Driscoll and Healy's (1994) sampling theorem.

Usage

-----

theta, phi = MakeGradientDH (cilm, [lmax, sampling, lmax_calc, extend, radius])

Returns

-------

theta : float, dimension (nlat, nlong)

A 2D map of the theta component of the horizontal gradient that conforms to

the sampling theorem of Driscoll and Healy (1994). If sampling is 1, the

grid is equally sampled and is dimensioned as (n by n), where n is 2lmax+2.

If sampling is 2, the grid is equally spaced and is dimensioned as (n by

2n). The first latitudinal band of the grid corresponds to 90 N, the

latitudinal sampling interval is 180/n degrees, and the default behavior is

to exclude the latitudinal band for 90 S. The first longitudinal band of the

grid is 0 E, by default the longitudinal band for 360 E is not included, and

the longitudinal sampling interval is 360/n for an equally sampled and 180/n

for an equally spaced grid, respectively. If extend is 1, the longitudinal

band for 360 E and the latitudinal band for 90 S will be included, which

increases each of the dimensions of the grid by 1.

phi : float, dimension (nlat, nlong)

A 2D equally sampled or equally spaced grid of the phi component of the

horizontal gradient.

Parameters

----------

cilm : float, dimension (2, lmaxin+1, lmaxin+1)

The real 4-pi normalized spherical harmonic coefficients of a scalar

function. The coefficients c1lm and c2lm refer to the cosine and sine

coefficients, respectively, with c1lm=cilm[0,l,m] and c2lm=cilm[1,l,m].

lmax : optional, integer, default = lmaxin

The maximum spherical harmonic degree of the coefficients cilm. This

determines the number of samples of the output grids, n=2lmax+2, and the

latitudinal sampling interval, 90/(lmax+1).

sampling : optional, integer, default = 2

If 1 (default) the output grids are equally sampled (n by n). If 2, the

grids are equally spaced (n by 2n).

lmax_calc : optional, integer, default = lmax

The maximum spherical harmonic degree used in evaluating the functions. This

must be less than or equal to lmax.

extend : optional, bool, default = False

If True, compute the longitudinal band for 360 E and the latitudinal band

for 90 S. This increases each of the dimensions of griddh by 1.

radius : optional, float, default = 1.0

The radius of the sphere used when computing the gradient of the function.

Description

-----------

MakeGradientDH will compute the horizontal gradient of a scalar function on a

sphere defined by the spherical harmonic coefficients cilm. The output grids of

the theta and phi components of the gradient are either equally sampled (n by n)

or equally spaced (n by 2n) in latitude and longitude. The gradient is given by

the formula

Grad F = 1/r dF/theta theta-hat + 1/(r sin theta) dF/dphi phi-hat.

where theta is colatitude and phi is longitude. The radius r is by default set

to 1, but this can be modified by use of the optional parameter radius.

The default is to use an input grid that is equally sampled (n by n), but this

can be changed to use an equally spaced grid (n by 2n) by the optional argument

sampling. The redundant longitudinal band for 360 E and the latitudinal band for

90 S are excluded by default, but these can be computed by specifying the

optional argument extend.

Reference

---------

Driscoll, J.R. and D.M. Healy, Computing Fourier transforms and convolutions on

the 2-sphere, Adv. Appl. Math., 15, 202-250, 1994.