Determine the spherical harmonic coefficients of a real function rotated by

three Euler angles.

Usage

-----

cilmrot = SHRotateRealCoef (cilm, x, dj, [lmax])

Returns

-------

cilmrot : float, dimension (2, lmax+1, lmax+1)

The spherical harmonic coefficients of the rotated function, normalized for

use with the geodesy 4-pi spherical harmonics.

Parameters

----------

cilm : float, dimension (2, lmaxin+1, lmaxin+1)

The input real spherical harmonic coefficients. The coefficients must

correspond to geodesy 4-pi normalized spherical harmonics that do not

possess the Condon-Shortley phase convention.

x : float, dimension(3)

The three Euler angles, alpha, beta, and gamma, in radians.

dj : float, dimension (lmaxin2+1, lmaxin2+1, lmaxin2+1)

The rotation matrix dj(pi/2), obtained from a call to djpi2.

lmax : optional, integer, default = lmaxin

The maximum spherical harmonic degree of the input and output coefficients.

Description

-----------

SHRotateRealCoef will take the real spherical harmonic coefficients of a

function, rotate it according to the three Euler anlges in x, and output the

spherical harmonic coefficients of the rotated function. The input and output

coefficients must correspond to geodesy 4-pi normalized spherical harmonics that

do not possess the Condon-Shortley phase convention. The input rotation matrix

dj is computed by a call to djpi2.

The rotation of a coordinate system or body can be viewed in two complementary

ways involving three successive rotations. Both methods have the same initial

and final configurations, and the angles listed in both schemes are the same.

This routine uses the 'y convention', where the second rotation axis corresponds

to the y axis.

Scheme A:

(I) Rotation about the z axis by alpha.

(II) Rotation about the new y axis by beta.

(III) Rotation about the new z axis by gamma.

Scheme B:

(I) Rotation about the z axis by gamma.

(II) Rotation about the initial y axis by beta.

(III) Rotation about the initial z axis by alpha.

The rotations can further be viewed either as a rotation of the coordinate

system or the physical body. For a rotation of the coordinate system without

rotation of the physical body, use

x(alpha, beta, gamma).

For a rotation of the physical body without rotation of the coordinate system,

use

x(-gamma, -beta, -alpha).

The inverse transform of x(alpha, beta, gamma) is x(-gamma, -beta, -alpha).

Note that this routine uses the "y convention", where the second rotation is

with respect to the new y axis. If alpha, beta, and gamma were originally

defined in terms of the "x convention", where the second rotation was with

respect to the new x axis, the Euler angles according to the y convention would

be alpha_y=alpha_x-pi/2, beta_x=beta_y, and gamma_y=gamma_x+pi/2.

This routine first converts the real coefficients to complex form using SHrtoc.

Then the coefficients are converted to indexed form using SHCilmToCindex, these

are sent to SHRotateCoef, the result if converted back to cilm complex form

using SHCindexToCilm, and these are finally converted back to real form using

SHctor.