%N passmake

%D Create passband calibration set

%P lgm

%: calibration

%B

PASSMAKE is a MIRIAD task that makes a baseline based passband

calibration dataset, to be used by PASSFIT and CALAPPLY.

%A vis

The passband calibrator visibility dataset.  Only one input

dataset allowed. This is normally an observation of a passband

calibrator (strong continuum source) observed in the same narrow-band

correlator mode as the science source; alternatively, it may be a

passband calibrator observed in the broad-band mode (correlator

mode 4, 40 MHz) provided the frequencies observed in broad-band

mode include the frequencies observed in the narrow-band observed

for the science source.

No default.

%A select

This keyword selects the subset of the visibility data to be

processed. There are a number of subcommands, which may be

abbreviated. Each may be prefixed with a plus or minus sign to

indicate using or discarding the selected data. Many subcommands

can be given (separate them by commas).

Examples:

'select=time(03OCT30:05:42:08,03OCT30:05:43:09),ant(7)(2)'

'select=-auto,-ant(7,8)'

'select=-ant(7,8),vis(1,15),vis(31,45),vis(301,1000)'

'select=-ant(7,8),-ant(1)(4,6),-ant(2)(4,5,6),-ant(3)(4,5),-ant(4)(5,6),-ant(5)(6)'

See also documentation for subroutine selprobe(), which may very well differ

slightly from this document.

Subcommands include:

time(t1,t2)

Select data between times t1 and t2 (UT). Times are in the format:

yymmmdd.fff

or

yymmmdd:hh:mm:ss.s

Various abbreviations are possible. If no date part is given, then

the time matchs all data (regardless of its date) for the given

time of day. e.g. time(03OCT30:05:41:08,03OCT30:05:43:09)

antennae(a1,a2,...)(b1,b2...)

Select all baselines pairs formed between first and second list of

antennas. The second list is optional and defaults to all antennas.

uvrange(uvmin,uvmax)

Select visibilities with uv radius between uvmin and uvmax (in kilo

wavelenghts). If only one value is given, uvmin is taken as zero.

uvnrange(uvmin,uvmax)

Select visibilities with uv radius between uvmin and uvmax (in

nanoseconds). If only one value is given, uvmin is taken as zero.

visibility(n1,n2)

Select visibilities numbered n1 to n2 inclusive.

increment(inc)

Select every inc'th visibility.

ra(r1,r2)

Select visibilities whose RA is in the range r1 to r2. RAs are given

in the format

hh:mm:ss

or

hh.hhh

Various abbreviations are possible.

dec(d1,d2)

Select visibilites whose DEC is in the range d1 to d2. Declinations

are given in the format

dd:mm:ss

or

dd.ddd

Various abbreviations are possible.

ha(h1,h2)

Selects on hour angle (values in decimal hours or hh:mm:ss)

lst(lst1,lst2)

Selects on LST (values in decimal hours or hh:mm:ss)

elevation(el1,el2)

Selects on elevation (values in degrees)

dra(p1,p2)

Select visibilities for which the RA of the pointing center is offset

from the main pointing center by between p1 and p2 arcseconds.

ddec(p1,p2)

Select visibilities for which the DEC of the pointing center is offset

from the main pointing center by between p1 and p2 arcseconds.

dazim(p1,p2)

Select visibilities for which the DAZIM offset is between p1 and p2 arcseconds.

delev(p1,p2)

Select visibilities for which the DELEV offset is between p1 and p2 arcseconds.

pointing(p1,p2)

Select visibilities with the rms pointing error in the range p1 to p2

arcseconds. If only one number is given, p1 is taken as 0.

polarization(p1,p2,p3,...)

Select visibilities on the basis of their polarization/Stokes parameter.

p1,p2,p3,... can be selected from "i,q,u,v,xx,yy,xy,yx,rr,ll,rl,lr"

Conversion between polarizations Stokes parameters (e.g. convert

linears to Stokes)  is not performed by this mechanism (see keyword stokes).

source(SRCNAM1,SRCNAM2,...)

Select correlations from the particular source. Several sources

can be given. An asterisk in the source name is treated as a

wildcard. Note that the sourcename match is done IGNORING CASE.

purpose(TYPE)

Select based on the intent/purpose of the observation. Introduced in

CARMA, where the letters B (bandpass), F (fluxcal), G (gaincal),

P (polarization), R (radio pointing), S (science) and O (other)

are used.

Multiple letters from this set can be chooses. No wildcards allowed.

seeing(r1,r2)

Select on seeing monitor rms path length (in microns). The UV variable used

is smonrms for ATCA, rmspath for CARMA

frequency(f1,f2)

Select correlations, where the sky frequency of the first correlation

is in the range f1 to f2 (in GHz). If only a single frequency is

given, then correlations are selected if the first correlation

is within 1% of the given frequency.  Note this selects the whole

record.  This may include several spectral-windows whose frequency

is quite different from the first correlation in the first window.

amplitude(amplo,amphi)

Select any correlation, where the amplitude is between "amplo" and

"amphi". If only one value is given, then "amphi" is assumed to be

infinite.

shadow(d)

Selects data that would be shadowed by an antenna of diameter "d" meters.

If "d" is zero, then the actual diameter of the antennas (if known) is used.

If some data is shadowed, it is advisable to use an antenna diameter value

greater than the physical antenna size (e.g. 20% larger).

See also the task CSFLAG for more finegrained approach for heterogeneous

arrays and dealing with sub arrays.

bin(b1,b2)

Select visibilities which have pulsar bin numbers in the range

b1 to b2 inclusive. If b2 is omitted, just pulsar bin b1 is

selected.

on(n)

This is used with single dish observations, anbd selects based

whether the "on" variable in the data is on (1) or off (0).

auto

This is used with files that contain a mix of autocorrelations and

crosscorrelations. This selects just the autocorrelation data.

window(w1,w2,...)

Select by spectral window (IF band in AIPS terminology). See

the help on window for more information.

or

The list of subcommands before and after the 'or' are "ored"

together.

and

Default ???

Default: all data selected.

%A gcal

The phase and amplitude gains solution to apply to data, usually

obtained from using calfit on broad-band observations of the passband

calibrator before and after the narrow-band observations; it is not

used,needed? for mode = self.

Default is to apply NO gains solution.

%A aver

Type of time-averaging to apply to passband data. Choices are ``vector'',

``scalar'' or ``self calibration''.  If self-calbration is selected,

the individual digital correlator channels are divided by the wideband

digital correlator channel AT EACH TIME INTERVAL, then vector averaged.

aver=Self-cal overrides user input for 'mode'.

lower sidebands.

Default: vector

%A mode

Method for normalizing the digital correlator channels relative to the

USB and LSB wide-band digital channels. Choices are ``cross'' and

``self''.  Cross mode normalizes the channels relative to the wide-band

data obtained  before and after the narrow-band passband observation,

using the gain file specified by gcal. The channel data are divided at

each time interval by  the interpolated fit and averaged according to

values of aver. Self mode  normalizes the channel data relative to the

wideband digital correlator by dividing the averaged channel data by the

averaged wideband data; the division is done after averaging according

to the value of aver.

Default is ``cross'' if a gcal file is specified.

Default is ``self'' is no gcal file is specified.

%A pcal

Name for output passband calibration data file.  This file may

be used as an input to calapply to calibrate the passband of

a dataset or passfit to interactively fit low order polynomials.

No default.