%N gfiddle

%D Fiddle with a (gain) visibility dataset

%P pjt

%: calibration

%B

GFIDDLE is a MIRIAD task that allows interactive and batch

processing of a gain visibility dataset (created by GMAKE).

It can also modify the flags of a regular visibility dataset

but adding breakpoints and fits has no meaning for them.

Also see UVWIDE on how to feedback wide band flags to narrow

band flags.

You can actually fiddle with *any* visibility dataset, so the

units could be K, Jy, Jy/K, or sqrt(Jy/K), depending on the

origin of the dataset, although the reported units may not

always come out as you think.

Gains can be either antenna based or baseline based.

By default, the single sideband data are displayed.  One can

optionally run in double sideband mode (i.e. phase differences

and amplitude ratios) by using the dsb option (see below).

GMAKE, when run to compute the double sideband solution, puts

the single sideband solutions (ssb) in the first two wide

channels and the double sideband solutions (dsb) in the third

and fourth channels.  If the dsb solutions are not present

and dsb mode is still requested, then then ssb data will be

used to compute the dsb data.

Commands in cursor mode are:

d   -- delete the nearest point (i.e. flag bad).

a   -- add the nearest deleted point (i.e. flag good).

x   -- zoom/unzoom toggle on the current column.

y   -- zoom/unzoom toggle on the current row.

z   -- zoom/unzoom toggle on the current plot.

b   -- insert a breakpoint.

c   -- delete the nearest breakpoint.

m   -- switch between DSB/SSB mode (see note with ssb option below).

0-9 -- order of the polynomial fit/number of spline nodes.

t   -- toggle between polynomial and cubic spline fits.

f   -- two-point fit (simple linear interpolation).

l   -- toggle between single/all baselines (see link option below).

i   -- info on the nearest (good) data point.

r   -- redraw the screen (also rescales the axes).

+   -- advance to the next page (if multiple pages).

-   -- backup to the previous page (if multiple pages).

<>  -- shift left (right) one panel (only in zoom mode).

v^  -- shift down (up) one panel (only in zoom mode).

?,h -- this help.

q   -- quit (no saving).

e   -- exit (save flags, breakpoints, and fits, if modified).

.      Toggle DEBUG mode.

NOTE: there is no confirmation on 'q' or 'e'.

%A vis

Name of the input uv dataset.  There is no default and a

dataset must be supplied.

%A line

The line to be selected. Default: wide,2

There can only be one or two channels selected; all others cause

the program to terminate with a warning.  Also, any other valid

option but the default will be permitted, but warned about.

Note that this will also affect changed flags, breakpoints, and

what is written to the optional output dataset.

This keyword is usually omitted by the user.

%A select

UV dataselection. Default: all

Note that this will also affect changed flags, breakpoints and

what is written to the optional output dataset.

This keyword is usually omitted by the user.

%A device

PGPLOT graphics device name. If none is supplied or the device

selected does not have an interactive cursor, then the program

will run in batch mode (i.e. no cursor interaction).  The default

device is /null (batch mode).

%A batch

Name of a batch file in which interactive (if the PGPLOT device

supports it) commands can be stored. By default commands are appended

to the file, but with 'options=new' this behavior can be changed.

Be careful when you re-apply batch files with commands that have a

cumulative action (flags, breaks etc.)

%A nxy

The maximum number of plots per page in the x direction.  The

number of plots in the y direction is controlled by the program.

Be careful if you use this keyword with batch= since they must

be consistent accross calls.

The default is 3 plots in the x direction.

%A fit

This keyword provides a way for the user to apply an initial fit

to the data.  The arguments to this keyword determines what kind

of initial fit will be applied (and possible optional parameters).

Valid options are:

poly,<amp_poly_order>,<phase_poly_order>

2pt

spline,<#_of_spline_nodes>

The amp and phase polynomial orders must be in the range [0, 9].

Omitting the phase polynomial order defaults to order 2; omitting

the amplitude order defaults to order 0.

The number of spline nodes must be in the range [0, 9] (the

actual number of spline nodes is one larger).  The default for the

spline fit argument is 0 (i.e. 1 node).  The 2pt fit simply

connects the dots; there are no additional parameters to this fit.

By default, no initial fit is applied.  However, the fit values

are initialized to 1 for amplitudes and 0 for phases.

%A sample

Output sample time in minutes.  It is adviced to set the sampling

time finer than the typical intervals in the input file.

The default is 1 minute.

%A clip

Automatically clips all amplitudes larger than this value.

The default is to perform no clipping.

%A options

Additional processing options. Multiple options, if compatible,

can be given, separated by commas.

link      While flagging points, this option applies the change

to the corresponding points on other baselines.

Similarly, with breakpoints.  Note that unflagging and

clearing of breakpoints cannot be done in this mode.

The interactive counterpart (key l) allows the user to

switch this mode interactively.

nowrap    Do not attempt to unwrap phases.

sources   If no break point information is present in the input

uv dataset, the program will automatically set break

points.  Focus changes always create a break point.

Source name changes will also create a break point

but only if this option is used.

nobreak   Do not set breakpoint on reading the data. By default

it will.

dsb       By default, the single sideband data are displayed.

If this option is present, then the plots and fits will

be done in double sideband mode (i.e. phase differences

and amplitude ratios).  Note that this option controls

which of the windows permits editing.  The interactive

counterpart (key m) permits viewing of the other window,

but no editing.

new       In batch mode an existing batch file will normally be

processed in a special append mode: first the existing

commands are read and applied, then new commands will

be added to the batch file. options=new will overwrite

the batch file and start from scratch.

debug     Debugging mode.  A lot of output to aid in providing

the programmer with relevant info. The '.' command

can be used as a debug toggle in interactive mode.

%A ampmax

If supplied, the amplitude scale is fixed from 0 to ampmax;

otherwise auto-scaling is done.

The default is to auto-scale the amplitudes.

%A phminmax

Range in phases to be displayed.  If supplied, two values (in

units of degrees) are needed.  If only one value is supplied,

the second value will be the negative of the first.

The default is to auto-scale the phases.

%A phase

List of (antenna,{U|L},phase_slope/hour) 3-tuples that

are used as an estimate for unwrapping the phases.  When low

signal/noise data have large phase drifts, the automatic phase

unwrapper may not be sufficient to unwrap the phase.  This

parameter is an attempt to give the user some automated way to

experiment with unwrapping phases.  Phases are given in degrees;

Antenna is an integer; and U/L is given as 1/2 (for upper or

lower sideband).

Default: not used.

%A out

The name of the output UV dataset. The default is to NOT write

out (and, hence, not save) the derived fits.

COLORS:

If the graphic device selected is capable of drawing with various

colors, then the following list will identify their meanings:

BLUE:

Fits drawn in blue indicate that no output uv dataset

name was provided.

GREEN:

If a valid output dataset is provided, then new fits

will be drawn in green.

YELLOW:

Yellow fits identify windows in which the fit is old

for some reason.  A new fit should be generated.