%N uvrandom

%D Generate random sampled uv points.

%P mchw

%: uv analysis

%B

Generate random sampled uv points from uniform sample between

(-umax,umax) and (-vmax,vmax) plus zero spacing point. This can

be useful for sampling a single dish model using uvmodel.

%A npts

Number of points to generate. Default=100.

%A nwide

Number of wideband channels. 1 or 0. Default=0.

%A nchan

Number of spectral channels. Default=100.

%A freq

Observing frequency in GHz. Default=100 Ghz.

%A inttime

Integration time per sample. Default is 100 seconds.

%A uvmax

Two values for maximum u and v measured in nanosecs. If only

one value given, this is used for both u and v. Default=1000,1000.

%A gauss

Use gaussian distribution instead of a uniform distribution of

visibilities. Default: false

%A noise

Gaussian noise in Jy written to the sampled uvdata.

Default is a constant visibility = 1 Jy.

%A out

The name of the output uv data set. No default.