%N uvaflag

%D Use flags in one visibility dataset to flag another.

%P rjs

%: calibration

%B

UVAFLAG is a MIRIAD task which flags correlations in one

visibility dataset based on the flag status of matching

correlations in a template dataset.

The template dataset should be a subset of the visibility dataset

being flagged, and the order of records in the two datasets

should be the same. Normally correlations in the template and

visibility datasets are matched by comparing time, polarisation,

baseline, bin and frequency. However options `nopol',`nofreq'

and 'nobin' can turn off the matching of polarisation, frequency

and bin  number.

%A vis

The input visibility file to be flagged. No default.

%A tvis

The template input visibility file. The default is the same

as the `vis' dataset. This default makes no sense without the

`nopol' or `nofreq' options. Several files can be given.

Wildcards are supported.

%A select

Normal visibility selection, which is applied to the template

dataset. See the help on "select" for more information.

%A options

Extra processing options. Several can be given, separated by

commas.

Minimum match is supported. Possible values are:

nopol  The polarisation of the records in the template are

ignored. If any polarisation in the template is

flagged, then all polarisations in the input are flagged

This can be useful when applying flagging based on one

polarisation type (e.g. stokes V) to the other

polarisations.

nofreq The frequency of the correlations in the template are

ignored. If any channel in the template is flagged, then

all channels in the input are flagged. This can be

useful when applying flagging based on a `channel-0'

dataset.

nobin  The bin numbers of the correlations in the template are

ignored. If any bin in the template is flagged, then

all bins in the input are flagged. This can be useful

when applying flagging based on a bin averaged dataset.

freq   The first baseline of the template is read and the

flags are applied to all times, baselines and bins

in the input. This can be useful when applying spectral

flags based on a small sample of data or time averaged

data. Option freq implies nobin. It can be combined

with nopol, but not with nofreq.

noapply Do not apply the flagging, just report the statistics

about what would be flagged.