%N mfclean

%D Multi-frequency synthesis CLEAN.

%P rjs

%: deconvolution

%B

MFCLEAN is a MIRIAD task to deconvolve a multi-frequency synthesis

image. It can perform either Clark or Hogbom iterations.

To achieve good results, the width of the beam should be at

least 3 times that of the region being cleaned. The

region being cleaned should be reasonably centred in the map, and

should have an appreciable guard band around it to the map edge (of

size comparable to the width of the region being cleaned).

To form a multi-frequency synthesis image and beam, use INVERTs

``mfs'' and ``sdb'' options. This will create

a map with one plane, and a beam with two planes (the normal dirty

beam, and the spectral dirty beam).

The result of MFCLEAN is a component image, consisting of two planes.

The first plane is the normal flux components. The second plane are

components of ``flux times spectral index'' (that is, I*alpha).

Use task MFSPIN to get a crude spectral index image from the output

of MFCLEAN.

The sign convention used for the spectral index, alpha, is that:

I(f) = I(f0) * (f/f0) ** alpha

MFCLEAN differs from CLEAN in a number of ways.

* Task CLEAN only requires that the beam is twice the size of the

region being cleaned whereas, for MFCLEAN, it is recommended that

the dirty beam be three times the size of the region being cleaned.

* MFCLEAN requires a guard band around the edge of the region being

cleaned.

* MFCLEAN does not have a Steer cleaning option, nor prussian hats.

%A map

The input dirty map, which should have units of Jy/beam. No

default.

%A beam

The input dirty beam. This should be formed using INVERT with

options=sdb. No default.

%A model

An initial model of the deconvolved image. This could be the

output from a previous run of MFCLEAN. It must have flux units of

Jy/pixel. The default is no model (i.e. a zero map).

%A out

The name of the output map. The units of the output will be

Jy/pixel. This file will contain the contribution of the input model.

It will consist of two planes, giving the flux density image and the

"flux times spectral index" image (also called the scaled flux

derivative image). No default.

%A gain

The minor iteration loop gain. Two values can be given, the second

being the gain for the spectral components. If only one value is

given, the flux and spectral components use the same gain. The

default is 0.1.

%A cutoff

MFCLEAN finishes when the absolute maximum residual falls below

CUTOFF. Default is 0.

%A niters

The maximum number of minor iterations. MFCLEAN finishes when

abs(NITERS) minor iterations have been performed. Clean may finish

before this point, however, if NITERS is negative and the absolute

maximum residual becomes negative valued, or if the cutoff level

(as described above) is reached.

%A region

This specifies the region to be Cleaned. See the Users Manual for

instructions on how to specify this. The default is generally

inadequate, and a smaller region should be explicitly specified.

%A minpatch

The minimum patch size when performing minor iterations. Default

is 51, but make this larger if you are having problems with

corrugations. You can make it smaller when cleaning images which

consist of a pretty good dirty beam.

%A speed

This is the same as the speed-up factor in the AIPS APCLN.

Negative values makes the rule used to end a major iteration more

conservative. This causes less components to be found during a

major iteration, and so should improve the quality of the Clean

algorithm. Usually this will not be needed unless you are having

problems with corrugations. A positive value can be useful when

cleaning simple point-like sources. Default is 0.

%A mode

This can be either "hogbom", "clark" or "any", and

determines the Clean algorithm used. If the mode is "any", then

MFCLEAN determines which is the best algorithm to use. The default

is "any".

%A log

Output log file containing a list of all the components. The log

file consists of 5 columns, being the iteration number, the x and

y pixel coordinate (in the output model; this goes from 1 to N),

the "I" component and the "I*alpha" component. The default is to not

create a log file.