%N atfix

%D Apply various miscellaneous corrections to ATCA visibility data

%P rjs

%: uv analysis

%B

ATFIX performs various miscellaneous offline corrections appropriate

for ATCA 3mm data. Steps that can be performed are

Apply gain/elevation effect

Apply system temperature measurements

Correct for incorrect baseline length and instrumental phase.

Correct antenna table (when some antennas were off-line)

NOTE: This task will usually be used very early in the data reduction

process. It should be used before calibration.

%A vis

The names of the input uv data sets. No default.

%A select

Standard visibility data selection. See the help on "select" for

more information. The default is to select all data.

%A out

The name of the output uv data set. No default.

%A dantpos

Currently the on-line system uses the antenna locations derived from

preliminary observations.

If a higher precision solution for the baseline lengths is available,

you will want to correct your data to account for this.

This also invokes corrections of known instrumental phase errors.

The inputs are the equatorial coordinate offsets entered in the

following order (NO checking is done for consistency):

dantpos = A1,X1,Y1,Z1,A2,X2,Y2,Z2,A3,X3,Y3,Z3,....

The input values are the antenna number and the three equatorial

coordinate offsets (entered in units of nanoseconds).  These input

values are added to the absolute coordinates read from the data.

Antenna present in the data but not included in the input value

list are treated as having a zero coordinate offset.

The arcane unit of nanoseconds is used for historical compatibililty.

Note 1 nanosec = 0.2997 meters.

A collection of parameter files giving the corrections to apply are

stored in $MIRCAT. These have names of the form "dantpos.yymmdd"

where "yymmdd" is the date of the start of a new array configuration.

Baseline corrections are believed to be constant between array

configurations.

If a data file is present, you can instruct atfix to read this

directly using the indirect parameter input. For example, to read

parameters appropriate for a hypothetical array configuration

starting on 16 October 2002, use

dantpos=@$MIRCAT/dantpos.021016

%A tsyscal

The determines the way that system temperature measurements are

applied to the visibility data. Usually this will be used

to make changes to the way system temperature measurements have

been applied to the data by the on-line system. The default

is to interpolate values between successive system temperature

measurements. Possible values

for this parameter are:

none        Do not apply any system temperature measurements. If the

the measurements were applied on-line, they are undone.

any         Do nothing. Leave the Tsys calibration as is.

constant    Apply a constant Tsys between successive Tsys measurements.

This is the approach used by the on-line system at 3mm

wavelength.

extrapolate Apply a Tsys value between measurements which accounts

for predicted changes in Tsys with elevation and measured

weather.

interpolate Interpolate a Tsys value between measurements which

accounts for predicted changes in Tsys with elevation

and measured weather. This is the default.

%A options

Extra processing options. Several options can be given,

separated by commas. Minimum match is supported.

nogainel  This disables applying a instrumental gain/elevation

correction to the data. Currently the gains of the

antennas are a function of elevation. By default,

a correction is made for gain/elevation effects.

noinst    Do not correct for instrumental baseline effects. By

default a phase correction is made for instrumental

baseline effects.

%A array

One of the flaws in the current ATCA datafiles is that antenna locations

are not recorded for antennas that are off-line (and hence not producing

data). While this might not seem a serious flaw, the off-line antennas

can still cause shadowing. In particular, this was an issue when

using the 3-antenna system in compact arrays.

By giving a value to this parameter, atfix will fill in any

antenna locations that are missing from the input visibility file.

NOTE: This just fills in missing antenna locations, it does not perform

any flagging of shadowed data.

The value given to this parameter is either an array configuration name

(e.g. EW352 or 750A) or a list of six station names (e.g. W106 or N3).

When giving the station names, these must be in the order of the antennas

(i.e. CA01, CA02, CA03 etc).

NOTE: When antennas are in a shuffled order, or for arrays using

the north spur, you should generally give the list of station

names, as the standard array configuration names assume the

standard antenna order (not the shuffled order).

If in doubt, see the on-line history of configurations:

http://www.narrabri.atnf.csiro.au/operations/array_configurations/config_hist.html