%N smafix

%D Plot and fit Tsys and do Tsys corrections.

%P jhz

%: plotting, analysis and uvdata correction

%B

SmaFix plots and does least square fitting with an order

of polynomial to the antenna-based Tsys measurements.

SmaFix also applies the Tsys corrections to the visibilies.

SmaFix can handle upto 100 sources per file. For options

of polynomial fitting to the data of individual sources

separately, the maximum number sources that can be handled

in SmaFix is 48.

%A vis

The name of the input data-set. No default.

%A out

The name of the output data-set with corrections. No default.

%A device

The PGPLOT plotting device to use. The default is no plot.

%A log

The log to give a listing of the variables. The default is no log.

%A xaxis

Variable to plot on the X axis. Currently supports

"time" and antel". Default is "time".

%A yaxis

Variable to plot on the Y axis. Default is "systemp".

Currently supports only "systemp".

%A nxy

Number of plots in the x and y directions. The default varies.

%A bant

An array of antenna ids of which the antennas

are corrupted in Tsys measurements.

example: bant=4, 8  indicates that  the Tsys values of both

the antennas 4 and 8 are corrupted.

%A fscal

An array of scaling factors to scale the Tsys values from

the good antenna (gant) to those of bad antennas (bant):

Tsys_bant(i) = fscal(i) * Tsys_gant.

Default is unity.

%A gant

A antenna id with good Tsys measurements which will

be used to replace the Tsys of the antennas assigend

with bant.

example: gant=1 assigns antenna 1 with good Tsys

to replace the Tsys values of antennas 4 and 8.

Defaults are no replacements.

%A xrange

The min and max range along the x axis of the plots. The default

is to autoscale. Note that for "time" should be given in normal Miriad

time format (either absolute time or time-of-day).

%A yrange

The min and max range along the y axis of the plots. The default

is to autoscale. Note that for "time" should be given in normal Miriad

time format (either absolute time or time-of-day).

%A dofit

A degree polynomial fit. Value 2 is for parabolic, 3 for cubic.

Default or a value of 0 and smaller corresponds to no fit to perform.

%A options

Extra processing options. Several can be given, separated by

commas. Minimum match is used.

"dtime"    If the x-axis is "time", plot the time in fractions

of a day. The default is to plot in hh:mm:ss.

"compress" If the variable is a 2D array of values, average

the variables along the second dimension.

"overlay"  By default, when a variable takes on several values,

each is plotted on a separate plot. The overlay

option makes a single plot.

"unwrap"   Unwrap the phases on the yaxis variable. Cannot unwrap

an xaxis variable. By default phases are not unwrapped.

"tsyscorr" If the y-axis is "systmp", Tsys correction will be

applied.

By default, no Tsys corrections are made.

"dosour"   If dofit is true, then the polynomial fit is source

dependent.

By default, no source separation is made.

"tsysswap" Swaps the values of systemp with a polynomial fit

in the Tsys corrections, saves the values from

the polynomial fit to replace the original

systemp and creates a new variable systmp to

save the original Tsys measurements.

For the cases of options=tsysfix or dofit > 0, gant > 0,

tsysswap is the defualt.

"all"      uses all data associated with flagged and unflagged

visibilties. The default uses only unflagged

(good) data.

"tsysfix"  uses the polynomial fit to Tsys from the unflagged (good)

data to interpolate the Tsys at flagged data points

and replaces only the bad Tsys values with the polynomial

fit in the case of the Keyword dofit = 1 or greater.

The flag state will be reset to 'unflag' for all the

visibilities except for those with illegal polarization

states. Warning: this options will also reset the flag

states passed from the SMA online system.

By default (both tsysswap and tsysfix are not chosen

in options), the original Tsys measurements will

be used in the Tsys corrections and the variables

from the input uvdata file remains unchanged if

options=tsyscorr is chosen.