%N imrm

%D Compute rotation measure image from position angle images

%P nebk

%: image analysis

%B

IMRM computes  rotation measure and zero wavelength position

angle images from at least 2 position angle images at

different frequencies.   This is done via a linear least

squares fit to:

PA = PA_0 + RM*LAMBDA**2

where RM is the rotation measure (rad/m**2) and PA_0 is the

position angle at zero wavelength.  The output rotation

measure image is in rad/m**2, and the output position

angle image is in degrees.  Optionally, plots of the fits

can be made.

The more frequencies you have the better.  It is very important

to try and get at least two sufficiently close that there is

no ambiguity between them.

By default, IMRM attempts to remove n*pi ambiguities from the

data.  Its algorithm is (pixel by pixel)

0) First remove angle according to the amount given by the

user (keyword "rmi") and the equation PA = RM*LAMBDA**2

1) Put the position angles of the first two frequencies

in the range +/- 90 degrees.

2) Remove 180 degree ambiguity from the position angles given

by the FIRST TWO IMAGES (keyword "in").  Thus, it modifies

the position angle of the second frequency by 180 degrees

so that the absolute value of the angle between the

two position angles is less than 90 degrees.

3) Compute the initial RM and PA_0 from these FIRST

TWO position angles.

4) This RM and PA_0 is used to predict the expected position

angle at the other frequencies according to the expression

PA = PA_0 + RM*LAMBDA**2.  Integer amounts of 180 degrees

are then added or subtracted to the position angles at the

remaining frequencies in order to make the position angle

as close as possible to the expected value.

5) Then a least squares fit is used to solve for the RM and PA_0

6) Finally, the procedure is repeated from step 0) where the

initial guess is now the value just determined above in

step 5).

The order in which the images are given is thus very important.

You should generally give your images in order of decreasing

frequency, with the assumption being that the smallest angle

between the first two represents a rough guess for the RM

with no ambiguities.  However, if you are very certain abou

the lack of ambiguity between certain frequencies, or there

are some of particularly high S/N and likely lack of ambiguity,

you may like to try these.  Its a nasty business and it is VERY

important that you look at the results carefully.

The attempt to remove ambiguities can be turned off with

keyword "options=ambiguous".  In this case, its algorithm is

0) First remove angle according to the intial guess given

by the user (keyword "rmi").

1) Put all position angles in the range +/- 90 degrees

2) Then a least squares fit is used to solve for the RM and PA_0

In principle, you should never need to use this option.

If there are no ambiguities, the first algorithm shouldn't

find any !

There are also a variety of methods offered with which to blank the

output images.  Most of these require error images associated with

the input position angle images. Use the program IMPOL to make

the position angle images and position angle error images.

%A in

Up to 5 input position angle (positive N -> E) images

(in degrees) at different frequencies.  Generally, you should

give the images in order of decreasing frequency.

Wild card expansion is supported, no default.

%A inerr

Up to 5 position angle error images (in degrees) used for

weighting the data during the least squares fit.  They are

assumed to be in one-to-one association with the position

angle images. If no error images are given, each position

angle image is given equal weight and we must assume a goodness

of fit of unity in order to find the output image errors.

Wild card expansion is supported, default is no error images.

%A rmi

An amount of rotation measure to remove from the data before fitting.

If you have a good idea of this, it helps enormously in removing

ambiguities. See the detailed use in the discussion of the algorithm

above.  See also options=guess where it is used slightly differently.

Default is 0

%A rm

Two values. The output fitted rotation measure image in

rad/m**2, and optionally, its associated error image.

The default is no output RM images.

%A pa0

The output fitted (at zero wavelength) position angle image

in degrees, and optionally, its associated error image.

The default is no output PA images.

%A qcut

Blank the output image (RM or PA) pixels if the goodness of fit

(Q) is less than this value.  If Q is larger than about 0.1 say,

the fit is believable.  If it is greater than 0.001, the fit

may be acceptable if the errors are non-normal or too small. If

Q is less than 0.001 the model can be called into question.  The

probability distribution for position angle images approximates

a Gaussian at high S/N ratios.  At low S/N ratios (roughly, when

P/sigma < 2) it is non-Gaussian.  If you don't specify error

images, Q cannot be determined and is assumed to be one.  This is

also true if you give IMRM position angle images at two

frequencies only.

Default is 0.001

%A errcut

Blank the output image (RM or PA) pixels if ANY of the input PA

image pixels has an error greater than this value (degrees).

Default is no input error based blanking.

%A rmcut

Blank pixels in BOTH the output RM and PA_0 images when the error

in the fitted RM is greater than this value (rad/m**2).

Errors can be worked out if you give input error images,

or if you input images at more than two frequencies AND we

assume the goodness of fit is unity.

Default is no fitted RM error based blanking.

%A pacut

Blank pixels in BOTH the output RM and PA_0 images when the

error in the fitted PA_0 is greater than this value (degrees).

Errors can be worked out if you give input error images,

or if you input images at more than two frequencies AND we

assume the goodness of fit is unity.

Default is no fitted PA_0 error based blanking.

%A device

PGPLOT plotting device to see the fits to the data.  The absolute

pixel numbers in x and y are also written into the corner of the

plot (unless options=accumulate).

No default.

%A nxy

Number of subplots per page in the x and y directions, to put

on the plotting device.  See options=accumulate

The default is 10x10

%A csize

PGPLOT character height.

Default is 1.0

%A options

Task enrichment options.  Minimum match is active,

"relax"      issue warnings instead of a fatal error when image

axis descriptors are inconsistent with each other,

and when the input image headers do not indicate that

they are position angle images (btype=position_angle)

"guess"      when removing ambiguities, this option causes IMRM to

use the rotation measure input through the keyword

"rmi" in step 3 above (on the first pass only), rather

than working it out from the first two frequencies. By

default, angle is removed from the data according to

the value of "rmi" and then the first guess made from

the first two frequencies.  The angle is not removed

in this way with this option.  This may prove useful if

you have two close but perhaps noisy frequencies which

is causing the initial guess of the RM to be wrong

(because of noise) and driving the subsequent turn

removal off.

"ambiguous"  Do not try to remove ambiguites.

"accumulate" means put all the plots on one sub-plot, rather than

the default, which is to put the plot for each

spatial pixel on a spearate subplot

"yindependent"

By default, the sub-plots are all drawn with the same

Y-axis scale, that embraces all sub-plots.  This option

forces each sub-plot to be scaled independently.