%N smauvamp

%D Analyses visibiliy amplitude by plotting data on a PGPLOT device.

%P jhz

%: uv analysis, plotting

%B

smauvamp - Analyses visibiliy amplitude by plotting visibility data

with error bars for 1 standard deviation in the mean

if averaging is invoked.

%A vis

The input visibility file(s). Multiple input files and wild card

card expansion are supported.

No default

%A line

This is the normal linetype specification. See the help on "line"

for more information. The default is all channels.

%A select

This selects which visibilities to be used. Default is all

visibilities. See the Users Guide for information about how

to specify uv data selection.

Default is all data

%A stokes

Select Stokes parameter(s) or polarization(s) from:

xx, yy, xy, yx,  i, q, u, v,

rr, ll, rl, lr

Default is all polarizations or Stokes parameters present

%A axis

Two values (minimum match active), one for each of the x

and y axes chosen from:

time                     [time in DD HH MM SS.S format]

dtime                    [time in decimal days format]

amplitude, real, imag    [natural units; Jy]

phase                    [degrees]

uc, vc                   [u,& v, -u & -v in klambda]

uvdistance               [sqrt(u**2+v**2)]

Defaults are axis=vc,amp  (x and y axes).

%A xrange

Plot range in the x-direction

If axis = uc, vc,               [kilo-lambda;   2 values]

unless OPTIONS=NANOSEC;     then   nanoseconds]

If axis = amplitude, real, imag [natural units; 2 values]

If axis = phase                 [degrees;       2 values]

If axis = time                  [dd,hh,mm,ss.s; 8 values]

If axis = dtime                 [decimal days;  2 values]

Default is to self-scale (see also OPTIONS=XIND).

%A yrange

Plot range in the y-direction as for the x axis.  The

default is to self-scale (see also OPTIONS=YIND).

%A average

The averaging time in minutes.

Averaging is reset at frequency, source, or pointing centre

changes. Individual baselines and polarizations are averaged

separately. If you have selected multiple

channels and you also ask for time averaging, then all the

selected channels are averaged together in the time interval.

The vector averaging for the visibility in a time interval

is done before data is binned if nbin > 0 is selected.

Default is no averaging.

%A nbin

Number of bins in xrange for xaxis.

If average > 0, the weighting in the bin statistics utilizes

the rms of the variables for axis = amplitude, real, imag and phase.

Default is no bin.

%A options

Task enrichment options. Minimum match is effective.

nocal   Do not apply the gain corrections

nopol   Do not apply the polarization leakage corrections

nopass  Do not apply the bandpass corrections

nofqav  By default, uvplt averages together all channels from

a visibility record before plotting. The nofqav option

disables this, and causes individual channels to be

plotted.

all     Plot flagged and unflagged visibilties

flagged Plot only flagged visibilities

The default is to plot only unflagged (good) visibilities

ALL overrides  FLAGGED

nanosec u and v are plotted in nano-seconds rather than k-lambda

xind    If the x-axis is self-scaled, then unless OPTIONS=NOBASE,

setting XIND will cause each sub-plot to have the x-axis

self-scaled independently.  The default is that the x-range

used is that which encompasses the ranges from all sub-plots.

yind    The equivalent for the y-axis

equal   Plot x and y with equal scales.  Useful only for plots

like AXIS=UU,VV.  Does not mean the plot will necessarily

be square

zero    Plot the x=0 and y=0 lines

symbols Each file is plotted with a different plot symbol

nocolour

Each file is plotted with the same colour (white). By

default and when there is only one polarization, each

file has a separate colour.

source  Put the source name rather than the file name in the

plot title

inter   After the plot is drawn, you get a chance to redraw

the plot with a different x- and y-range, and also

on a different device.  In this way you can make a

hard-copy without re-running the program. In this case,

points outside of the user specified x,y-range are

ARE included in the plot buffer, so that if you redefine

log     Write the values  that are

plotted into the log file.

%A PosA

The Position Angle (degree) for rotating the u-v plane.

If PA is not zero, the uv frame will rotates amount of

PA around the w axis in the case of xaxis=uc or vc.

%A device

PGPLOT plot device/type. No default.

%A size

PGPLOT character sizes, in units of the default size (i.e., 1)

First value is for the labels, the second is for the symbol size

Defaults depend upon the number of sub-plots. The second value

defaults to the first.

%A log

The output logfile name. The default is the terminal.

%A comment

A one line comment which is written into the logfile.