%N pnt

%D Display and Fit pointing data.

%P mchw

%: pointing analysis

%B

PNT is an interactive program for plotting, fitting and editing

pointing data obtained from total power measurements or voltage

pattern measurements with the interferometer.    The program is

directed by the commands below, and writes a log file  PNT.LOG,

and an ascii file of the final pointing constants, PNTFIT.date,

which is updated if the program is restarted.  This file can be

used to change the pointing constants used at Hat Creek.

%A device

PGPLOT display device. Default is to prompt the user.

%A telescop

telescope name for telescope dependent pointing data format.

known telescopes are ATA BIMA OVRO and CARMA

default is telescop=CARMA with data format. Note telescop

is case sensitive and we use the upper case versions here.

%A pdevice

Hardcopy plot device in the format plot#/pgplot device.

# increments for sucessive plots. Default is 'plot1/ps'

%A log

Output log file. Default is 'pnt.log'

%A options

Special processing options. [Y/N]. Default=N.

COMMANDS

IN    Read multiple files of  pointing data  written by the

programs SPOINT or CROSS. The program starts by asking

for the data files to be used.  Subsequent use of this

command replaces the data to be reduced.  The data are

edited to the pointing constants in the first pointing

record read. Special options can suppress this editing,

or remove the old pointing corrections from the data.

LI    List the pointing constants, and optionally, the data.

GR    Plot  DAZ  DEL  AZ  EL or TILT versus  AZ  EL or UT.

The sources to be plotted may be selected and plotted

with different symbols. The plot can be manipulated

using CURSOR OPTIONS.

CO    Enter comment into log file.

FL    Fit flux densities to radio data. (See code for format)

HI    Histogram plot of pointing errors.

FI    Fit pointing constants to azimuth or elevation errors.

Points deleted in the plots are not included in the fit.

Up to 9 parameters are fitted in a least squares sense

to the following 4 equations. The Default is equation 3.

Equation 1 has tilt and sin(2*az), cos(2*az) terms:

daz = v(1)*cosel + v(2) + v(3)*sinel + sinel*(v(4)*sin(az)

+ v(5)*cos(az)) + cosel*(v(6)*sin(2.*az) + v(7)*cos(2.*az))

Equation 2 has sin(az) and cos(az) and 2*az terms:

daz = v(1)*cosel + v(2) + v(3)*sinel + cosel*(v(4)*sin(az)

+ v(5)*cos(az)) + cosel*(v(6)*sin(2*az) + v(7)*cos(2*az))

Equation 3 has tilt and sin(az), cos(az) terms:

daz = v(1)*cosel + v(2) + v(3)*sinel +sinel*(v(4)*sin(az)

+ v(5)*cos(az)) + cosel*(v(6)*sin(az) + v(7)*cos(az))

The pointing constants represent the following:

v(1) - Azimuth encoder + antenna mount offset from the

meridian.

v(2) - Collimation error of optical or radio axis from

mechanical axis (orthogonal to the elevation axis).

The optical and radio pointing will differ in this term.

v(3) - Elevation axis misalignment w.r.t. azimuth axis.

v(4) to v(7) - Tilt and azimuth axis errors.

Equations 1 to 3 fit the elevation pointing to:

del = v(1) +v(2)*sin(el) + v(3)*cos(el) + v(4)*sin(az)

+ v(5)*cos(az) + v(6)*sin(2*az) + v(7)*tan(1.5708-el)

v(1) - Elevation encoder offset.

v(2) and v(3) - Elevation errors. The radio pointing

also includes the subreflector sag.

v(4) and v(5) - tilt of azimuth axis.

v(6) - Fourier analysis residual.

v(7) - Refraction. Radio and optical pointing differ.

Equation 4 fits tilt and sin and cos az and 2az terms

in a different order:

daz = v(1)*cosel + sinel*(v(2)*sin(az) + v(3)*cos(az))

+ v(4) + v(5)*sinel + cosel*(v(6)*sin(az) + v(7)*cos(az))

+ cosel*(v(8)*sin(2*az) + v(9)*cos(2*az))

del = v(1) + v(2)*sin(az)) + v(3)*cos(az) + v(4)*sin(el)

+ v(5)*cos(el) + v(6)*sin(2*az) + v(7)*cos(2az)

+ v(8)*tan(1.5708-el)

Equation 5 fits Gerry's daz = v(5) * sin(El) * cos(El) for ATA - 09mar2005

daz = v(1)*cosel + sinel*(v(2)*sin(az) + v(3)*cos(az))

+ v(4) + v(5)*sinel*cosel + cosel*(v(6)*sin(az) + v(7)*cos(az))

+ cosel*(v(8)*sin(2*az) + v(9)*cos(2*az))

del = v(1) + v(2)*sin(az)) + v(3)*cos(az) + v(4)*sin(el)

+ v(5)*cos(el) + v(6)*sin(2*az) + v(7)*cos(2az)

+ v(8)*tan(1.5708-el)

Although the functions in these equations are not orthogonal,

additional corrections may be added to the pointing constants

already in use provided that sufficient pointing data are taken

to separate the terms.  The correlation matrix for each fit,

indicates the degree of independence of the parameters fitted.

Avoid OVERFITTING THE DATA. Fitting more parameters will

improve the rms for any data set but a large sample of data

points is needed to obtain significant results. After

moving the antennas only the azimuth offset and tilt should change.

The other pointing parameters should not be fitted with small

data sets.

ED    Edit the pointing data, either to the constants fitted

above, or to another set entered from the keyboard.

SOURCE SELECTION

Plotting and fitting commands request a selection of sources.

"A" includes all sources. <CR> ends the source selection.

CURSOR OPTIONS

The plotting programs allow manipulation of the data directed

by the cursor position and typing one character to select

the option. A group of data points can be selected by L(eft),

R(ight), T(op), and B(ottom). V(anish) removes points from the

plot and subsequent fits.  A(ppear) restores all the data.

I(dentify) labels the data. W(rite) lists the labels. N(ew) sets

new limits for the plot. M(ore) adds points to a plot. S(ee)

replots the data and P(rinter) makes a printer plot. H(elp)

lists the cursor options.