%N smavarfit

%D Analyse correlations of antenna gains and uv-variables.

%P jhz/mchw

%: uv analysis

%B

SMAVARFIT is a Miriad task to analyse the correlations of

antenna gains and uv-variables rooted from the Miriad task

VARFIT. In addition to  fitting the following:

yaxis = slope *  xaxis +  intercept

D(yaxis) = slope * D(xaxis) + intercept

where D() is the difference between succesive samples,

SMAVARFIT provides an option of  regression for

the phases derived from the gains of two uvfiles.

If either axis is an antenna gain, then SELFCAL must be run first

and the axes are sampled at the interval used in SELFCAL.

The rms and correlation coefficient are listed for each antenna.

%A vis

The input UV dataset name. The antenna gains must first be

derived for the selected linetype by the task SELFCAL with

the desired averaging interval.

%A xaxis

xaxis can be antenna gain ``amplitude'' or

``phase'', or any uv-variables. The default is ``time''.

%A yaxis

yaxis can be antenna gain ``amplitude'' or

``phase'', or any uv-variables. The default is ``phase''.

%A log

Output log file. Default is the terminal.

%A device

Standard PGPLOT device, if plot of yaxis versus xaxis is wanted.

Default is no plot.

%A nxy

Number of plots in the x and y directions. The default is 3,3.

%A xrange

The min and max range along the x axis of the plots. The default

is to autoscale. Note that for "time" should be given in days.

%A yrange

The min and max range along the y axis of the plots. The default

is to autoscale. Note that for "time" should be given in days.

%A refant

Reference antenna for gains and uv-variables. If refant.ne.0 then

the gain of this antenna is set to cmplx(1.,0.). The

other antenna gains are then relative to the reference antenna.

The uv-variable value for the refant is subtracted from each of the

other antennas. Single valued variables, e.g. time, are unchanged.

The default is to use the original gains and uv-variable values.

%A refant2

Reference antenna for cross correlation of gains and uv-variables.

Compare the gains or uv-variables between antennas by fitting

yaxis = slope * xaxis(refant2) + intercept

D(yaxis) = slope * D(xaxis(refant2)) + intercept

where D() is the difference between succesive samples.

%A options

wrap         Do not unwrap phase.

xscale       Re-scale xaxis = slope * xaxis + offset, where slope

and offset are fitted w.r.t. xaxis(refant)

The fit is written into the log file. E.g.

yaxis=phase xaxis=tpower refant=4 log=tpscale

results can be used in the task tpgains to correct

tpower to a common Tsys scale.

yscale       Re-scale yaxis = slope * yaxis + offset, where slope

and offset are fitted w.r.t. yaxis(refant)

structure    Replace yaxis with it's structure function:

<{yaxis(i+k) - yaxis(i)}**2>

allan        Replace yaxis with it's Allan variance:

<{yaxis(i+2k) - 2*yaxis(i+k) + yaxis(i)}**2>

quad         Fit yaxis = a + b*xaxis + c*xaxis**2

where a and b are first derived from a linear fit.

The fit is written into the log file. E.g.

yaxis=tpower xaxis=tpower refant2=4 options=quad

results can be used in the task tpgains to correct

tpower to a common Tsys scale.

phareg       do linear regression between phase1 and phase2 derived

from the gain tables in uvfile1 and uvfile2, respectively:

phase2 = slope * phase1 + offset