%N gpboot

%D Correct the gains table of a visibility data-set.

%P rjs

%: calibration

%B

GpBoot is a MIRIAD program which corrects a visibility data-set's

gain table.

GpBoot works by comparing the amplitudes in the gain table of one

data-set with those of another. The gains are assumed to differ

by a constant factor. It is assumed that the

instrumental gain and atmospheric attenuation of the two data-sets

were the same, and so the difference in gain is due to an

arbitrary or incorrect flux being specified when initially

determining the gains.

%A vis

The input visibility file, containing the gain file to correct.

The gains are assumed to be out by a constant factor.

%A cal

This is a visibility data-set, which is assumed to contain a

gain table which scales the data to absolute flux units.

%A select

Normal uv-selection parameter. This selects the gains in the

``vis'' that are compared against the ``cal'' gains (note that

all the ``cal'' gains are involved in the comparison). Currently

ONLY time and antenna selection are permitted. You will use

this parameter to select which data the instrumental/atmospheric

amplitude gains for ``vis'' are comparable to the observation in

``cal''.