%N smablflag

%D Interactive flagging task.

%P jhz

%: uv analysis

%B

SmaBLFLAG is an interactive flagger. SmaBLFLAG plots the visibilities

(e.g. time vs amplitude or antel vs systemp) either a baseline at a

time or all at once, and allows you to flag discrepant points using

the plotting cursor. The sources and polarization states are color-coded.

There are a few simple flagging commands, which  you enter as a single

character at the keyboard. The following commands

are possible:

Left-Button  Left mouse button flags the nearest visibility.

Right-Button Right mouse button causes SmaBLFLAG to precede to the

next baseline.

<CR>         Carriage-return gives help.

?            Help.

a            Flag nearest visibility (same as left mouse button).

c            Clear the flagging for this baseline, and redraw plot.

h            Give help (same as carriage return).

p            Define a polygonal region, and flag visibilities

within this region. You define the vertices of the

polygon by moving the cursor and then hitting the

left mouse button (or a). You finish defining the

polygon by hitting the right mouse button (or x).

You can delete vertices with the middle mouse

button (or d).

q            Abort completely. This does not apply flagging.

r            Redraw plot.

u            Unzoom.

x            Move to next baseline (same as right mouse button).

z            Zoom in. You follow this by clicking the mouse on the

left and right limits to zoom.

The visibility data are color-coded for the program sources.

%A vis

Input visibility dataset to be flagged. No default.

%A line

The normal Miriad linetype specification. SmaBLFLAG will average

all channels together before displaying them, and any flagging

that you do will be applied to all selected channels. The default

is all channels.

%A device

Normal PGPLOT plotting device. It must be interactive. No default.

%A stokes

Normal Stokes/polarisation parameter selection. The default

is `ii' (i.e. Stokes-I assuming the source is unpolarised).

NOTE SmaBLFLAG plots the average of all the selected Stokes/polarisation

quantities. Also it flags ALL quantities, regardless of whether they

were selected or not.

%A select

Normal visibility data selection. Only selected data can be

flagged. The default is to select all data.

%A axis

Two character strings, giving the X and Y axes of the plot. Possible

axis values are:

time         (the default for the X axis)

lst          Local apparent sidereal time.

antel        antenna elevation in degree;

only for systemp flagging.

uvdistance   sqrt(u**2+v**2)

hangle       (hour angle)

amplitude    (the default for the Y axis)

phase

real

imaginary

systemp      (only for Y axis)

%A options

Task enrichment parameters. Several can be given, separated by

commas. Minimum match is used. Possible values are:

nobase  Normally SmaBLFLAG plots a single baseline at a time.

This option causes all baselines to be plotted on

a single plot.

selgen  Generate a file appropriate for selecting the bad

data (via a "select" keyword). The output is a text

file called "blflag.select".

noapply Do not apply the flagging.

rms     When processing spectra, blflag normally plots the

mean value of the spectra. Using options=rms causes

if to plot the rms value instead.

scalar  When processing spectra, blflag normally forms an

average value by vector averaging. The "scalar" option

causes it to generate the scalar average. This option

should be used with significant caution.

The following options can be used to disable calibration.

nocal   Do not apply antenna gain calibration.

nopass  Do not apply bandpass correction.

nopol   Do not apply polarisation leakage correction.